1. Connect to the Nexus switch directly via serial console using TeraTerm or PuTTY. Use Cisco Blue cable if available.

2. Power off switch by removing power cables all of them, removing one will not power off switch. "Note there is no power switch"

3. Power on the switch

4. On the TeraTerm or PuTTY press Crrl + j  and hold until break sequence to get to the switch(boot)# prompt.

5. Enter configure terminal command or config t

6. Set password type admin-password samplepassword << type customer password

7. Type exit

8. Type dir bootflash: as shown below

switch(boot)# dir bootflash:

4096  Jun 19 2019 06:25:34  .patch/

4096  Feb 05 2020 22:33:08  .rpmstore/

4096  Feb 05 2020 22:31:38  .swta/

3272  Jun 19 2019 06:40:28  20190619_064027_poap_13033_init.log

301001  Jun 19 2019 13:29:19  20190619_124720_poap_13031_init.log

2097156  Feb 05 2020 17:31:48  20200117_224401_poap_13030_1.log

1923551  Feb 05 2020 20:14:53  20200117_224401_poap_13030_2.log

1048600  Jan 18 2020 00:04:24  20200117_224401_poap_13030_init.log

16817  Feb 05 2020 23:33:05  NX3132Q_RCF_v1.3-Cluster-HA.txt

8080  Feb 05 2020 20:32:47  NX3132_RCF_v1.1_24p10g_26p40g.txt

22107  Feb 05 2020 21:33:43  NX3232_RCF_v1.4-Cluster-HA-Breakout.txt

0  Feb 05 2020 22:31:24  bootflash_sync_list

4096  Feb 05 2020 22:31:37  logflash/

4096  Feb 05 2020 22:24:28  lost+found/

698260992  Jun 19 2019 06:27:42  nxos.7.0.3.I4.1.bin

1006938112  Feb 05 2020 22:23:56  nxos.7.0.3.I7.5a.bin  <<< latest frimware NXOS or what customer is curently useing

0  Feb 05 2020 22:35:30  platform-sdk.cmd

4096  Jun 19 2019 06:40:14  scripts/

1024  May 10 2020 02:20:06  sprom_2_0_1

1024  May 10 2020 02:20:06  sprom_3_0_0

4096  Jun 19 2019 06:39:44  virtual-instance/

9. Boot the system image, example;

switch(boot)# load bootflash:nxos.7.0.3.I7.5a.bin

10. Log in to the switch with username admin and the new password.

11. Reset the new password to ensure that is it is also the SNMP password

switch# configure  terminal

switch(config)# username admin password <new password>

switch(config)# exit

switch#

12. Save configuration type  copy running-config startup-config